https://www.youtube.com/watch?v=dxk6Lons6ls&list=PLDOLa06NDxoK96T4xxouEr0k0HqXjs1WI&index=11

Perfect 👍 Let’s create your complete and neatly formatted CN Viva Preparation Notes — with all repeated questions merged and detailed, simple explanations (ideal for viva answers).
I’ll format this clearly for quick reading and memorization.

🌐 Computer Networks Viva Preparation (CN Viva Notes)

1. Full Form of TELNET

TELNET → Telecommunication Network

2. What is Telnet? Explain Telnet (with uses & disadvantages)

Definition:
Telnet is a network protocol used to remotely access and manage devices or computers over a TCP/IP network.
It allows a user to log in to another computer using a virtual terminal connection.

Working Principle:

- Operates on TCP port 23.

- Provides a command-line interface for communication with remote systems.

- Sends data as plain text between client and server.

Uses:

- Remote server administration.

- Network device configuration.

- Testing ports and connectivity.

Disadvantages:

- No encryption → data (including passwords) is sent in plain text.

- Insecure for the Internet (use SSH instead).

- Vulnerable to eavesdropping and spoofing attacks.

OSI Layer:

- Works at Application Layer.

3. What is Switching? What are its Types?

Definition:
Switching is a technique used to connect multiple devices in a network and transfer data efficiently between them.

Types of Switching:

- Circuit Switching

  - A dedicated path is established between sender and receiver before communication.

  - Example: Telephone networks.

  - Suitable for voice communication.

- Packet Switching

  - Data is divided into packets and sent individually over the network.

  - Each packet may take different routes.

  - Example: Internet (TCP/IP).

- Message Switching

  - Entire message is treated as one unit and transferred from node to node.

  - Store-and-forward mechanism.

  - Slower but no need for a dedicated path.

4. Explain the Operating Principle of FTP

FTP (File Transfer Protocol) is used to transfer files between client and server on a network.

Working:

- Works on TCP port 20 (data) and 21 (control).

- Uses two separate connections:

  - Control Connection → for commands (like login, directory change).

  - Data Connection → for actual file transfer.

- Supports uploading, downloading, and directory management.

OSI Layer:

- Application Layer

5. What is TCP? (TCP ke bare me batao)

Full Form: Transmission Control Protocol

Definition:
TCP is a connection-oriented, reliable, and byte-stream protocol used for communication between devices over the Internet.

Features:

- Provides error detection and correction.

- Ensures data sequencing (in correct order).

- Provides flow control and congestion control.

- Used in applications like HTTP, FTP, SMTP, etc.

OSI Layer:

- Transport Layer

6. Difference between TCP and UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Full Form | Transmission Control Protocol | User Datagram Protocol |
| Type | Connection-oriented | Connectionless |
| Reliability | Reliable (acknowledgements, retransmissions) | Unreliable (no acknowledgement) |
| Speed | Slower (overhead for reliability) | Faster |
| Order | Packets arrive in order | Packets may arrive out of order |
| Example Use | Web browsing, emails, file transfer | Video streaming, gaming, VoIP |
| Header Size | 20 bytes | 8 bytes |

7. Difference between TCP/IP and OSI Model

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Developed by | ISO | DoD (Department of Defense, USA) |
| Layers | 7 Layers | 4 Layers |
| Layer Names | Application, Presentation, Session, Transport, Network, Data Link, Physical | Application, Transport, Internet, Network Access |
| Protocol Dependency | Model-based, theoretical | Protocol-based, practical |
| Example Protocols | Not specific | HTTP, FTP, TCP, IP |
| Usage | Reference model for understanding | Practical implementation used on the Internet |

8. OSI Model Layers and Their Functions

| Layer No. | Layer Name | Functions / Units |
| --- | --- | --- |
| 7 | Application | User interface, network services (HTTP, FTP, SMTP) |
| 6 | Presentation | Data formatting, encryption, compression |
| 5 | Session | Establishes and maintains sessions |
| 4 | Transport | Reliable delivery (TCP, UDP) |
| 3 | Network | Routing, IP addressing |
| 2 | Data Link | Error detection, framing (MAC address) |
| 1 | Physical | Transmission of bits (cables, switches) |

Units used at each layer:

- Physical → Bits

- Data Link → Frames

- Network → Packets

- Transport → Segments

- Application/Session/Presentation → Data

9. TCP/IP Model Layers

| Layer | Protocols / Description |
| --- | --- |
| Application | HTTP, FTP, SMTP, Telnet |
| Transport | TCP, UDP |
| Internet | IP, ICMP, ARP |
| Network Access | Ethernet, Wi-Fi |

10. What are Headers?

Definition:
Headers are additional pieces of information added to data at each layer of the OSI/TCP model for control, addressing, and error handling.

Example:
When an email is sent —
Each layer adds its own header (IP header, TCP header, etc.) to help route and deliver the message.

11. What Headers are Used While Sending Emails?

- Application Layer: SMTP Header (contains sender, receiver, subject, etc.)

- Transport Layer: TCP Header (port numbers, sequence numbers)

- Network Layer: IP Header (source and destination IP addresses)

- Data Link Layer: Frame Header (MAC addresses)

12. What is SMTP?

Full Form: Simple Mail Transfer Protocol

Definition:
SMTP is a protocol used to send emails between mail servers.

Features:

- Uses TCP port 25 (or 587 for secured).

- Works in Application Layer.

- Handles only outgoing emails (sending part).

- For receiving, IMAP or POP3 is used.

13. What is Subnetting?

Definition:
Subnetting is the process of dividing a large IP network into smaller, manageable sub-networks (subnets).

Purpose:

- Efficient IP address management.

- Reduces network traffic.

- Improves security and performance.

Example:
If you have 192.168.1.0/24 → you can create 4 subnets with /26 each (64 IPs each).

14. What is a Default Gateway?

Definition:
A default gateway is the router or device that connects a local network to the outside network (Internet).
It acts as the exit point for traffic leaving your network.

Example:
If your IP is 192.168.1.10 → your gateway might be 192.168.1.1.

15. What are Common Error Detection Techniques?

1. Parity Bit:
Adds a bit to make number of 1s even or odd. (Simple, detects single-bit errors)

2. Checksum:
Sum of all bytes; receiver recomputes and compares. (Used in TCP/IP)

3. Cyclic Redundancy Check (CRC):
Polynomial division of data; very accurate for burst error detection.

4. Hamming Code:
Used for both error detection and correction.

16. What are Error Correction Methods in TCP?

- Automatic Repeat Request (ARQ):

  - Stop and Wait ARQ

  - Go-Back-N ARQ

  - Selective Repeat ARQ

- Checksum for error detection

- Acknowledgement (ACK/NACK) for reliability

- Retransmission on missing packets

17. What is Cisco Packet Tracer?

Definition:
Cisco Packet Tracer is a network simulation tool developed by Cisco Systems used to design, configure, and test network topologies virtually.

Features:

- Simulates routers, switches, PCs, cables, and IoT devices.

- Visualizes packet flow.

- Great for learning Cisco Networking (CCNA).

18. Types of Cables

| Cable Type | Description | Usage |
| --- | --- | --- |
| Straight Cable | Connects different devices (PC to Switch) | Used in LAN |
| Cross Cable | Connects similar devices (PC to PC, Switch to Switch) | Used for direct connections |
| Coaxial Cable | Copper core with shielding | Used in cable internet |
| Fiber Optic Cable | Uses light signals, very high speed | Backbone connections |
| UTP Cable (Cat 5/5e/6) | Unshielded twisted pair | Common in LANs |

19. What is Crimping?

Definition:
Crimping is the process of attaching RJ45 connectors to the ends of network cables (like Cat5 or Cat6) using a crimping tool.

Steps:

- Strip the cable.

- Arrange wires as per T568A or T568B standard.

- Insert wires into RJ45 connector.

- Crimp using tool to fix the pins.

✅ Tip for Viva:
Be confident and concise — when they ask follow-ups (like “where does Telnet work?”), always mention Layer name and port number.

Would you like me to generate this as a PDF (formatted neatly for print or phone revision)?

Sure 👍 — Let’s go through TCP, UDP, IPv4, and IPv6 headers one by one with diagrams + explanations (in concise and clear form).

🧩 1. TCP Header

Diagram:

0                   15 16                  31

+-------------------+-----------------------+

|     Source Port   |    Destination Port   |

+-------------------+-----------------------+

|               Sequence Number             |

+-------------------------------------------+

|           Acknowledgment Number           |

+---------------+-------+-------------------+

| Data Offset   |Flags  |   Window Size     |

+---------------+-------+-------------------+

|   Checksum    |Urgent Pointer            |

+---------------+---------------------------+

|        Options (if any)       | Padding   |

+-------------------------------------------+

|              Data (Payload)              |

+-------------------------------------------+

Field Explanation:

| Field | Description |
| --- | --- |
| Source Port / Destination Port | Identify sending and receiving applications. |
| Sequence Number | Number of the first data byte in this segment. |
| Acknowledgment Number | Next byte expected from the receiver. |
| Data Offset | Length of TCP header (in 32-bit words). |
| Flags | Control bits like SYN, ACK, FIN, RST, PSH, URG. |
| Window Size | Amount of data sender is willing to receive. |
| Checksum | Error-checking of header and data. |
| Urgent Pointer | Indicates urgent data. |
| Options | Used for parameters like MSS, window scaling, etc. |
| Data | Application data (payload). |

✅ TCP is reliable, connection-oriented, provides sequencing and error recovery.

🧭 2. UDP Header

Diagram:

0                   15 16                  31

+-------------------+-----------------------+

|     Source Port   |    Destination Port   |

+-------------------+-----------------------+

|        Length     |       Checksum        |

+-------------------+-----------------------+

|              Data (Payload)              |

+-------------------------------------------+

Field Explanation:

| Field | Description |
| --- | --- |
| Source Port / Destination Port | Identify sending and receiving processes. |
| Length | Total length of UDP header + data. |
| Checksum | Error detection for header and data. |
| Data | Actual data sent by the application. |

✅ UDP is connectionless, faster, and used for real-time or small data (e.g., DNS, video streaming).

🌐 3. IPv4 Header

Diagram:

0                   15 16                  31

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

|Version| IHL |Type of Service|        Total Length             |

+---------------------------------------------------------------+

|       Identification        |Flags|   Fragment Offset         |

+---------------------------------------------------------------+

|   Time to Live  | Protocol  |        Header Checksum          |

+---------------------------------------------------------------+

|                     Source IP Address                         |

+---------------------------------------------------------------+

|                   Destination IP Address                      |

+---------------------------------------------------------------+

|                 Options (if any) + Padding                    |

+---------------------------------------------------------------+

|                          Data                                 |

+---------------------------------------------------------------+

Field Explanation:

| Field | Description |
| --- | --- |
| Version | Always 4 for IPv4. |
| IHL (Internet Header Length) | Header length (in 32-bit words). |
| Type of Service (ToS) | Defines packet priority. |
| Total Length | Entire packet length (header + data). |
| Identification / Flags / Fragment Offset | Used for fragmentation and reassembly. |
| TTL (Time To Live) | Limits packet lifetime (decrements at each hop). |
| Protocol | Indicates next-level protocol (e.g., TCP=6, UDP=17). |
| Header Checksum | Error-check for header only. |
| Source/Destination Address | 32-bit IPv4 addresses. |
| Options | Optional parameters. |

✅ IPv4 uses 32-bit addresses → about 4.3 billion unique addresses.

🌍 4. IPv6 Header

Diagram:

0                   15 16                  31

+---------------------------------------------------------------+

|Version| Traffic Class |           Flow Label                  |

+---------------------------------------------------------------+

|        Payload Length         | Next Header | Hop Limit       |

+---------------------------------------------------------------+

|                   Source Address (128 bits)                   |

+---------------------------------------------------------------+

|                Destination Address (128 bits)                 |

+---------------------------------------------------------------+

|                          Data                                 |

+---------------------------------------------------------------+

Field Explanation:

| Field | Description |
| --- | --- |
| Version | Always 6 for IPv6. |
| Traffic Class | Packet priority (like ToS). |
| Flow Label | Identifies a flow of packets for QoS. |
| Payload Length | Length of data after header. |
| Next Header | Type of next header (e.g., TCP=6, UDP=17). |
| Hop Limit | Replaces TTL, decremented per hop. |
| Source/Destination Address | 128-bit addresses (vastly larger address space). |
| Data | Actual payload. |

✅ IPv6 simplifies header structure and eliminates checksum, fragmentation at routers, etc.

⚖️ Summary Table:

| Protocol | Header Size | Reliable? | Connection Type | Address Size | Main Use |
| --- | --- | --- | --- | --- | --- |
| TCP | 20–60 bytes | ✅ Yes | Connection-Oriented | — | Web, Email |
| UDP | 8 bytes | ❌ No | Connectionless | — | Streaming, DNS |
| IPv4 | 20–60 bytes | — | — | 32-bit | General Internet |
| IPv6 | 40 bytes | — | — | 128-bit | Modern Internet |

Would you like me to generate a single labeled diagram (image) that shows all four headers side by side for easy memorization (useful for viva or notes)?

Perfect — this is a core viva topic in Computer Networks 👇
Let’s break it down clearly into TCP vs UDP and IPv4 vs IPv6 🔍

⚙️ 1. TCP vs UDP

Both are Transport Layer protocols (Layer 4 in OSI, Layer 3 in TCP/IP).
Their main job is to enable communication between applications on different devices.

🔹 TCP (Transmission Control Protocol)

- Connection-oriented protocol → connection must be established before data transfer.

- Reliable → ensures all packets reach destination in correct order.

- Performs error checking, flow control, congestion control.

- Slower, due to overhead of reliability mechanisms.

📦 Examples: Web (HTTP/HTTPS), Email (SMTP), File transfer (FTP)

🔹 UDP (User Datagram Protocol)

- Connectionless protocol → sends packets (datagrams) without setup.

- Unreliable → no guarantee of delivery or order.

- No flow or congestion control.

- Faster, low latency — good for real-time applications.

📦 Examples: Video streaming, VoIP, DNS, Online gaming

⚖️ Difference Between TCP and UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Full Form | Transmission Control Protocol | User Datagram Protocol |
| Connection Type | Connection-oriented | Connectionless |
| Reliability | Reliable (acknowledgements, retransmission) | Unreliable (no ACKs, no retransmit) |
| Speed | Slower | Faster |
| Error Checking | Yes (via checksum, sequence numbers) | Minimal checksum only |
| Order of Packets | Maintains order | Order not guaranteed |
| Header Size | 20 bytes (min) | 8 bytes |
| Flow & Congestion Control | Yes | No |
| Applications | HTTP, FTP, SMTP, Telnet | DNS, VoIP, Video streaming, Gaming |

🌐 2. IPv4 vs IPv6

Both are Internet Layer protocols (Network Layer in OSI).
Their job is to assign IP addresses and route packets between devices.

🔹 IPv4 (Internet Protocol version 4)

- Uses 32-bit address → allows ~4.3 billion unique addresses.

- Address example: 192.168.0.1

- Written in decimal and dot-separated (four octets).

- Supports manual or DHCP configuration.

- Includes checksum in header.

🔹 IPv6 (Internet Protocol version 6)

- Uses 128-bit address → allows 3.4 × 10³⁸ unique addresses (almost infinite).

- Address example: 2001:0db8:85a3:0000:0000:8a2e:0370:7334

- Written in hexadecimal and colon-separated.

- Supports auto-configuration (stateless).

- No checksum in header (handled by upper layers).

- Includes better security and faster routing.

⚖️ Difference Between IPv4 and IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Length | 32 bits | 128 bits |
| Address Format | Decimal, dotted (192.168.1.1) | Hexadecimal, colon-separated (2001:db8::1) |
| No. of Addresses | ~4.3 billion | ~3.4 × 10³⁸ |
| Header Size | 20–60 bytes | Fixed 40 bytes |
| Checksum | Present | Not present |
| Security | Optional (IPSec optional) | Mandatory (IPSec built-in) |
| Configuration | Manual or DHCP | Auto or DHCPv6 |
| Broadcast | Supported | Not supported (uses multicast) |
| Routing | Complex | Simplified |
| Adoption | Widely used | Slowly replacing IPv4 |

💡 Quick Summary:

- TCP = Reliable + Ordered + Connection-oriented

- UDP = Fast + Unreliable + Connectionless

- IPv4 = Older, limited addresses (32-bit)

- IPv6 = Newer, larger address space (128-bit) with better efficiency

Would you like me to draw and label the TCP, UDP, IPv4, and IPv6 headers next (for your viva diagrams)? It’s a frequently asked question right after this one.